Workaround A20 vs. I/O Port 92h Bugs

Alas, from the no-good-deed-goes-unpunished file, it turns out that on

some systems the very act of writing to I/O port 92h (the actual value

doesn't matter) gates A20 on, but then that's all she wrote.  No

further writes to 92h or to the 8042 for that matter have any effect.

As this causes us some problems, we need to recognize this situation

and avoid it.  On the system I encountered with this hardware bug, the

input value of I/O port 92h is 0C0h, so the test used checks for the

two high-order bits on.  If that's a valid I/O port (at least in MCA

parlance), that would mean that both Fixed Disk lights are on, which

is, I trust, an unlikely event.  This also catches the case where the

I/O port has no effect and a read returns 0FFh.

SWAT_A20.ASM	 Check for invalid value from I/O port 92h before

writing to it.