ZyXEL Prestige 100IH and 100IH Plus

ZyNOS v2.41(G.01)  | 11/15/2000

Release Notes & Manual Supplement

Date: November 15, 2000

This Release Note/Manual Supplement contains information about the features, and bug fixes.

## Supported Platforms

V2.41(G.01) supports Prestige model: P100IH/P100IH Plus S/T and U interfaces.

*Note: Since the default configuration for ZyNOS 2.41 is not compatible with the one for the previous ZyNOS firmware, so if your P100IH is upgraded from the previous ZyNOS versions, please also update the default configuration file for ZyNOS 2.41.

## New Features:

CLID callback support for dial-in users

Dial-in user is capable for callback in Version 2.41.

Outgoing data call bumping support

Prestige will drop a channel in an MP bundle if there is a packet to other remote node.

DHCP relay

Prestige could be a middle role between DHCP server and DHCP client.

More DHCP users

The maximum IP address managed by DHCP server is 253.

NetMeeting support for SUA/NAT

NetMeeting is supported for both incoming and outgoing call.

Minimum toll period support

Prestige will try to use all the toll period at most with minimum toll period.

Time and Date Setting

Three time protocols Daytime (RFC-867), Time (RFC-868) and NTP (RFC-1305) are supported.

Call Scheduling

Prestige can allow the user to schedule a dial-up connection in specified time.

NAT(Network Address Translation)

Prestige supports four types NAT for IP packets: One to One, Many to One (current SUA), Many to Many Overload, and Many to Many No Overload.

Others

- The ICMP discovery protocol is turned off by default ROM file, and if users want to turn on this protocol to let users’ workstation (including PC) to recognize the P100IH as one of default route, then users should follow the procedure below.

- Go to CI command mode (menu 24.8 or menu 24.4.22)

- sys edit autoexec.net

- Continue pressing n until finding the string as “ip icmp discovery enif0 off”

- Press d to delete.

- Press x to save. It will work at next boot up.

- In order to make ICQ 99a to receive file behind SUA, you should do the following procedure.

- open ICQ preference in ICQ icon.

- open connections slot.

- In “internet connect type” select “I am using a permanent internet connection(LAN)”

- choose “I am behind a firewall or proxy”

- enter firewall settings. Modify firewall time out to 80 seconds.

- If you run NetMeeting program behind SUA to connect an outside user, the outside user will see two identical users in screen.

- Add CI command : PPP LCP ACFC ON/OFF.

- Support ISDN embedded protocol analyzer (EPA).

- “isdn fw analyzer on” turn on EPA.

- “isdn fw analyzer off” turn off EPA

- “isdn fw analyzer dump” dump EPA raw data, it need another program to analyze it.

## Enhancement Details

- CLID callback support for dial-in users

Functional Description

CLID is an authentication method to identify dial-in user. CLID callback is used for toll saving on ISDN because the call is disconnected immediately without picking up the phone. In previous version, only remote node is capable for CLID callback because there is no outgoing information for dial-in users. In 2.40, the CLID outgoing information will be set in Menu 13, and dial-in user is capable for callback.

SMT Changes

In Menu 13, login and password is not only for mutual authentication but also for outgoing callback.

Menu 13 - Default Dial-in Setup Telco Options: IP Address Supplied By: CLID Authen= None Dial-in User= Yes IP Pool= No PPP Options: IP Start Addr= N/A Recv Authen= CHAP/PAP IP Count(1,2)= N/A Compression= Yes Mutual Authen= Yes O/G Login= p100 Session Options: O/G Password= ******** Edit Filter Sets= No Multiple Link Options: Max Trans Rate(Kbps)= 128 Callback Budget Management: Allocated Budget(min)= Period(hr)= Press ENTER to Confirm or ESC to Cancel:

New CI command

No new CI command.

- Outgoing data call bumping support

Functional Description

Call bumping is a feature which Prestige will manage a MP bundle dynamically for different traffic.  That means Prestige will drop a channel in a bundle when necessary, and reconnect it when possible.  The current implementation works for POTS call only.  For data packet, Prestige won’t drop a channel for it if all channels are occupied.  With outgoing data call bumping, Prestige will drop a channel in an MP bundle if there is a packet to other remote node.

SMT Changes

No changes.

New CI command

No new CI command.

- DHCP relay

Functional Description

DHCP stands for Dynamic Host Configuration Protocol.  It includes three types of roles, DHCP server, DHCP relay and DHCP client.  DHCP server is a server who manages the IP addresses to DHCP clients.  DHCP relay is a middle role between server and client.  Whenever DHCP client request an IP address, DHCP relay will forward the request to a DHCP server and forward the response to the DHCP client from the DHCP server.  We have supported DHCP server in 1.40.  And DHCP relay will be supported in 2.40.

SMT Changes

New option will be added in 3.2 for DHCP relay.

Menu 3.2 - TCP/IP and DHCP Ethernet Setup DHCP= Relay Configuration: Client IP Pool Starting Address= N/A Size of Client IP Pool= N/A Primary DNS Server= N/A Secondary DNS Server= N/A Relay Server Address= 0.0.0.0 TCP/IP Setup: IP Address= 192.168.4.1 IP Subnet Mask= 255.255.255.0 RIP Direction= Both Version= RIP-2B Press ENTER to Confirm or ESC to Cancel: Press Space Bar to Toggle.

New CI command

| Ip | dhcp | iface name | mode | server/relay/none | set DHCP mode for server/relay/none |
| --- | --- | --- | --- | --- | --- |
|  |  |  | relay | server <ipaddr> | set DHCP relay server address |

- More DHCP users

Functional Description

The current IP addresses managed by DHCP server is 32 at maximum.  In 2.40, the maximum will be increased to 253 at least.

SMT Changes

No changes.

New CI command

No new CI command.

- NetMeeting support for SUA

Functional Description

In current version, NetMeeting does not work while SUA is enable.  In 2.40, NetMeeting will be supported for both incoming and outgoing call.  For outgoing call, there is no special setting.  For incoming call, port 1503 and 1720 is required to set for server.

SMT Changes

No changes.

New CI command

No new CI command.

- Minimum Toll Period external specification

Functional Description

Phone call is charged by time in most places.  It is always rounded up at the charging period.  For example, Prestige may be idled out and drop the call at 10 seconds in a 3 minutes toll period.  That call is still charged for 3 minutes.  That means the left 2 minutes and 50 seconds is wasted.  With minimum toll period, Prestige will try to use all the toll period at most.  In above case, Prestige will try to extend the idle timeout to the nearest 3 minutes.  If there is any packet during the extended 2 minutes and 50 seconds, the idle timeout will be cleared and the second call is eliminated.  Since the session time calculation by Prestige is not necessary synchronized with Telecom, Prestige will drop the channel 5 seconds shorter than the edge of toll period.  Therefore, the minimum toll period won’t work if the toll period is less than 5 seconds.

SMT Changes

Menu 11.1 - Remote Node Profile Rem Node Name= ? Edit PPP Options= No Active= Yes Rem IP Addr= ? Call Direction= Both Edit IP= No Incoming: Telco Option: Rem Login= ? Transfer Type= 64K Rem Password= ? Allocated Budget(min)= Rem CLID= Period(hr)= Call Back= No Nailed-Up Connection= N/A Outgoing: Toll Period(sec)= 0 My Login= Session Options: My Password= ******** Edit Filter Sets= No Authen= CHAP/PAP Idle Timeout(sec)= 100 Pri Phone #= ? Sec Phone #= Press ENTER to Confirm or ESC to Cancel:

New CI command

No new CI command.

- Time and Date Setting

Functional Description

During boot procedure, Prestige will try to connect to a time server somewhere in Internet to calibrate its system clock. In menu 24.10, Prestige administrator can choose which time protocol is preferred, where is the time server (i.e. its IP address) and the time zone of where Prestige is. The Prestige supports three time protocols, which are “Daytime (RFC-867)”, “Time (RFC-868)” and “NTP (RFC-1305)”. The former two protocols employ TCP protocol, and the last one use UDP protocol. Valid date value is from January 1, 1990 to February 5, 2036. The “Time Zone” field is referred to the time zone of where Prestige is located. It is NOT to the time server’s location.

SPECIAL NOTE: Daytime server offers reference time referred to server’s own local time, but not Greenwich Mean Time (GMT). So, the “Time Zone” field DOES NOT apply to this time service. The other two kinds of time service offer GMT reference time.

SMT design

Menu 24.10 - System Maintenance - Time and Date Setting Use Time Server when Bootup= None/Daytime/ Time/NTP Time Server IP Address= 202.132.154.169 Current Time: 14 : 00 : 47 New Time (hh:mm:ss): 14 : 00 : 47 Current Date: 1999 - 12 - 21 New Date (yyyy-mm-dd): 1999 - 12 - 21 Time Zone= GMT+0800 Press ENTER to Confirm or ESC to Cancel:

New CI command

| sys | adjtime | calibrate system clock with time server |
| --- | --- | --- |

- Call Scheduling

Functional Description

Prestige can allow the user to schedule a dial-up connection in specified time. It can let the ISP or the remote nodes in menu 11 to connect or to be dropped down automatically. User may configure a remote node with multiple schedules on a specified day or weekdays. Just like what the scheduled recording function in a video recorder, you can schedule a remote node to connect in a specified date and time.

SMT design

Menu 26 ‘Schedule Setup’ is added in Main Menu.

Copyright (c) 1994 – 1999 ZyXEL Communications Corp. Prestige 100IH Main Menu Getting Started Advanced Management 1. General Setup 21. Filter Set Configuration 2. ISDN Setup 3. Ethernet Setup 23. System Password 4. Internet Access Setup 24. System Maintenance Advanced Applications 26. Schedule Setup 11. Remote Node Setup 12. Static Routing Setup 13. Default Dial-in Setup 14. Dial-in User Setup 15. NAT Setup 99. Exit Enter Menu Selection Number:

There are twelve individual schedule sets in Menu 26.

Menu 26 - Schedule Setup Schedule Schedule Set # Name Set # Name ------ ----------------- ------ ----------------- 1 hinet 7 _______________ 2 _______________ 8 _______________ 3 _______________ 9 _______________ 4 _______________ 10 _______________ 5 _______________ 11 _______________ 6 _______________ 12 _______________ Enter Schedule Set Number to Configure= 1 Edit Name= hinet Press ENTER to Confirm or ESC to Cancel:

- Select a schedule number first.

- Give it a name and enter menu 26.1.

(P.S. If clear the ‘Name’ field to blank, Prestige will delete this schedule.)

Menu 26.1 is the schedule configuration

Menu 26.1 Schedule Set Setup Active= Yes/No Start Date(yyyy-mm-dd)= 1990 - 1 - 1 How Often= Once/Weekly Once: Date(yyyy-mm-dd)= 1990 - 1 - 1 Weekdays: Sunday= Yes/No or N/A Monday= Yes/No or N/A Tuesday= Yes/No or N/A Wednesday= Yes/No or N/A Thursday= Yes/No or N/A Friday= Yes/No or N/A Saturday= Yes/No or N/A Start Time(hh:mm)= 12 : 00 Duration(hh:mm)= 16 : 00 Action= Forced On/Forced Down/Disable Dial-on-demand /Enable Dial-on-demand Press ENTER to Confirm or ESC to Cancel:

- Active: schedule will be run or not.

- Start Date: valid from this day; doesn’t mean the first running day.

- How Often: selecting ‘Once’, and the schedule will be executed only on the ‘Once’’Date’. If selecting ‘Weekly’, it will run on selected weekdays during each week.

- Start Time: hour and minute for start this schedule

- Duration: how long will this schedule continue?

- Action

- Forced On: during the period, the remote node will always keep online (idle timeout will be disabled).

- Forced Down: during the period, this remote node will always keep down, If this remote node is already connected, then it will be dropped. Demanded dial or trigger call from LAN will be ignored.

- Enable Dial-on-demand: Enable Dial-on-demand: during the period, this remote node will allow to connect. D.O.D. is default to a remote node without schedules. This scheduled action can be used to override the other actions, like ‘Forced Down’.

- Disable Dial-on-demand: during the period, this remote node will deny any demand dial. If there are any connection online, will not drop it. After that connection is dropped manually or by idle timeout, remote node can't be triggered up then.

- When out of duration, scheduler will do nothing to this remote node. If a remote node with schedule ‘Forced On’ is over, it will return to normal operation ‘dial-on-demand’, not opposite ‘Forced Down’.

You can assign 1~4 schedule sets to a remote node in menu 11.1

Menu 11.1 - Remote Node Profile Rem Node Name= Hinet Edit PPP Options= No Active= Yes Rem IP Addr= 192.168.50.22 Call Direction= Outgoing Edit IP= No Incoming: Telco Option: Rem Login= N/A Transfer Type= 64K Rem Password= N/A Allocated Budget(min)= Rem CLID= N/A Period(hr)= Call Back= N/A Schedules= 1,3,2,11 Outgoing: Nailed-Up Connection= No My Login= abcd Toll Period(sec)= 0 My Password= **** Session Options: Authen= CHAP/PAP Edit Filter Sets= No Pri Phone #= 4125678 Idle Timeout(sec)= 100 Sec Phone #= Press ENTER to Confirm or ESC to Cancel:

New CI command

No new CI command.

- NAT(Network Address Translation)

Functional Description

NAT only applies to IP packets.  NAT will change a packet's IP address and port number to allow networks which do not have legal Internal address to access the Internet.  NAT has four types: One to One, Many to One, Many to Many Overload, and Many to Many No Overload.  The current SUA is considered NAT type Many to One.  Multiple NAT entries can be set up to handle different IP address ranges on the LAN.

NAT types Many to One and Many to Many Overload use Port Address Translation (PAT).  PAT utilizes port number and IGA(Internal Global Address: The outside IP address) to determine the ILA(Internal Local Address: The inside IP address).  Each IGA can be shared by multiple ILAs.

Each remote node can be assigned one NAT set.  When NAT is turned on, the rules in this set will be applied to all traffic through this remote node.  One IP address per remote node can be designated as the SUA Server IP address.  Packets received by the SUA Server can be redirected to an inside server as defined by the user.

SMT design

The menu 15 is changed from “SUA Server Setup” to “NAT Setup”.

Copyright (c) 1994 - 1999 ZyXEL Communications Corp. Prestige 100IH Main Menu Getting Started Advanced Management 1. General Setup 21. Filter Set Configuration 2. ISDN Setup 3. Ethernet Setup 23. System Password 4. Internet Access Setup 24. System Maintenance Advanced Applications 26. Schedule Setup 11. Remote Node Setup 12. Static Routing Setup 13. Default Dial-in Setup 14. Dial-in User Setup 15. NAT Setup 99. Exit Enter Menu Selection Number:

Menu 15 - NAT Setup 1. Address Mapping Sets 2. NAT Server Sets Enter Menu Selection Number:

Menu 15.1 - Address Mapping Sets 1. hinet 2. 3. 4. 5. 6. 7. 8. 255. SUA (Read Only) Enter Set Number to Edit:

Menu 15.1.1 - Address Mapping Rules Set Name= hinet Idx Local Start IP Local End IP Global Start IP Global End IP Type --- --------------- --------------- --------------- --------------- ------ 1. 0.0.0.0 255.255.255.255 0.0.0.0 M-1 2. Server Set= 1 0.0.0.0 Server 3. 4. 5. 6. 7. 8. 9. 10. Action= Edit , Select Rule= 0 Press ENTER to Confirm or ESC to Cancel:

Note:

- 4 actions are “Edit”, “Insert Before”, “Delete”, and “Save Set”; the default is “Edit”. “Edit” means      to edit the selected rule. “Insert Before” means to insert a rule before the select rule and all the rules after the selected rule will be drawn back one rule. “Delete” means to delete the selected rule and all the rules after the selected one will advance one rule. “Save Set” means to save the whole set and when users choose the action as that, “Select Rule” item will be disabled.

- The system processes the rules by turn.

Menu 15.1.1.1 - hinet - Rule 1 Type: Many-to-One Local IP: Start= 0.0.0.0 End = 255.255.255.255 Global IP: Start= 0.0.0.0 End = N/A Server Mapping Set= N/A Press ENTER to Confirm or ESC to Cancel:

Menu 15.1.1.2 - hinet - Rule 2 Type: Server Local IP: Start= N/A End = N/A Global IP: Start= 0.0.0.0 End = N/A Server Mapping Set= 1 Press ENTER to Confirm or ESC to Cancel:

Note:

- Total 5 types, “One-to-One”, “Many-to-One”, “Many-to-Many Overload”, “Many-to-Many No Overload” and “Server”.

- Whichever Local IP or Global IP, the end address must be after the start address.

- If the rule is for all local IP, put start address as 0.0.0.0 and end address as 255.255.255.255.

- If dynamic IP, put the value of Global Start IP as 0.0.0.0.

- The server mapping set is mapped in menu 15.2.

Menu 15.2 - NAT Server Sets 1. Server Set 1 2. Server Set 2 3. Server Set 3 4. Server Set 4 5. Server Set 5 6. Server Set 6 7. Server Set 7 8. Server Set 8 9. Server Set 9 10. Server Set 10 Enter Set Number to Edit:

Menu 15.2.1 - Multiple Server Configuration Port # IP Address ------ --------------- 1. Default 0.0.0.0 2. 0 0.0.0.0 3. 0 0.0.0.0 4. 0 0.0.0.0 5. 0 0.0.0.0 6. 0 0.0.0.0 7. 0 0.0.0.0 8. 0 0.0.0.0 9. 0 0.0.0.0 10. 0 0.0.0.0 11. 0 0.0.0.0 12. 0 0.0.0.0 Press ENTER to Confirm or ESC to Cancel:

Remote node setting is in menu 11.3 and menu 4

Menu 11.3 - Remote Node Network Layer Options Rem IP Addr: 0.0.0.0 Rem Subnet Mask= 0.0.0.0 My WAN Addr= 0.0.0.0 NAT= Full Feature Address Mapping Set= 1 Metric= 2 Private= No RIP Direction= Both Version= RIP-2B Press ENTER to Confirm or ESC to Cancel:

Menu 4 - Internet Access Setup ISP's Name= hinet Pri Phone #= 4125678 Sec Phone #= My Login= icchung My Password= ******** My WAN IP Addr= 0.0.0.0 NAT= [None/SUA Only/Full Feature] Address Mapping Set= 1 Telco Options: Transfer Type= 64K Multilink= Off Idle Timeout= 300 Press ENTER to Confirm or ESC to Cancel:

Note: To leave “My WAN IP Addr” as 0.0.0.0 means dynamic IP.

New CI command

| ip | nat | iamt |  | Display NAT system information. |
| --- | --- | --- | --- | --- |
|  |  | iface | <iface name> | Show the NAT status of an interface. |
|  |  | lookup | <set #> | List all the active rules in a set. |
|  |  | new-lookup | <set #> | List all the new rules which are not yet used due to conflicts with old rules. |
|  |  | reset | <iface name> | This deletes all the entries inside the session table and IAMT table but not the lookup table. |
|  |  | server |  | Display server table. |
|  |  | update |  | Updates the runtime NAT data to what is in ROM. |

- Examples

Case 1 (ISP case, internet access only)

In ISP case with internet access only, we need one rule that all ILAs share one dynamic IGA assigned by ISP for outgoing packet.

Enter to menu 15.1 and choose a set to create an address mapping set named ISP (the first set is used in this case).

Menu 15.1 - Address Mapping Sets 1. 2. 3. 4. 5. 6. 7. 8. 255. SUA (Read Only) Enter Set Number to Edit:

In menu 15.1.1, give the set a name (ISP in this case), then toggle action to edit and select rule 1 to edit.

Menu 15.1.1 - Address Mapping Rules Set Name= ISP Idx Local Start IP Local End IP Global Start IP Global End IP Type --- --------------- --------------- --------------- --------------- ------ 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. Action= Edit , Select Rule= 0 Press ENTER to Confirm or ESC to Cancel:

Toggle type to many-to-one because more than one machines share 1 IP in the most ISP case, enter local start IP as 0.0.0.0 and local end IP as 255.255.255.255 to mean this rule is used by all machines in local LAN, and enter global IP as 0.0.0.0 to mean the IGA is a dynamic IP.

Menu 15.1.1.1 - ISP - Rule 1 Type: Many-to-One Local IP: Start= 0.0.0.0 End = 255.255.255.255 Global IP: Start= 0.0.0.0 End = N/A Server Mapping Set= N/A Press ENTER to Confirm or ESC to Cancel:

After doing that, the user will watch the menu 15.1.1 as following:

Menu 15.1.1 - Address Mapping Rules Set Name= ISP Idx Local Start IP Local End IP Global Start IP Global End IP Type --- --------------- --------------- --------------- --------------- ------ 1. 0.0.0.0 255.255.255.255 0.0.0.0 M-1 2. 3. 4. 5. 6. 7. 8. 9. 10. Action= Edit , Select Rule= 0 Press ENTER to Confirm or ESC to Cancel:

Toggle action to Save Set to save all the set.

Menu 15.1 - Address Mapping Sets 1. ISP 2. 3. 4. 5. 6. 7. 8. 255. SUA (Read Only) Enter Set Number to Edit:

Edit menu 4 as following,

Menu 4 - Internet Access Setup ISP's Name= ISP Pri Phone #= 4125678 Sec Phone #= My Login= icchung My Password= ******** My WAN IP Addr= 0.0.0.0 NAT= [None/SUA Only/Full Feature] Address Mapping Set= 1 Telco Options: Transfer Type= 64K Multilink= Off Idle Timeout= 300 Press ENTER to Confirm or ESC to Cancel:

Case 2 (ISP case, with internal server)

In ISP case, we need to add one extra rule in menu 15.1.1 as one server rule for incoming packet. Firstly, go to menu 15.2, and select a NAT server set to edit (set 1 in this case), and then edit the default value as 192.168.1.10 shown as below.

Menu 15.2 - NAT Server Sets 1. Server Set 1 2. Server Set 2 3. Server Set 3 4. Server Set 4 5. Server Set 5 6. Server Set 6 7. Server Set 7 8. Server Set 8 9. Server Set 9 10. Server Set 10 Enter Set Number to Edit:

Menu 15.2.1 - Multiple Server Configuration Port # IP Address ------ --------------- 1. Default 192.168.1.10 2. 0 0.0.0.0 3. 0 0.0.0.0 4. 0 0.0.0.0 5. 0 0.0.0.0 6. 0 0.0.0.0 7. 0 0.0.0.0 8. 0 0.0.0.0 9. 0 0.0.0.0 10. 0 0.0.0.0 11. 0 0.0.0.0 12. 0 0.0.0.0 Press ENTER to Confirm or ESC to Cancel:

Secondly, go to menu 15.1.1.2 (rule 1 is used for outgoing packet), and then toggle type to Server, and insert global IP as 0.0.0.0 (dynamic IP) and server mapping set as 1.

Menu 15.1.1.2 - ISP - Rule 2 Type: Server Local IP: Start= N/A End = N/A Global IP: Start= 0.0.0.0 End = N/A Server Mapping Set= 1 Press ENTER to Confirm or ESC to Cancel:

Then the user can watch the menu 15.1.1 as following,

Menu 15.1.1 - Address Mapping Rules Set Name= ISP Idx Local Start IP Local End IP Global Start IP Global End IP Type --- --------------- --------------- --------------- --------------- ------ 1. 0.0.0.0 255.255.255.255 0.0.0.0 M-1 2. Server Set= 1 0.0.0.0 Server 3. 4. 5. 6. 7. 8. 9. 10. Action= Edit , Select Rule= 0 Press ENTER to Confirm or ESC to Cancel:

Toggle action to Save Set to save all the set.

Menu 4 - Internet Access Setup ISP's Name= ISP Pri Phone #= 4125678 Sec Phone #= My Login= icchung My Password= ******** My WAN IP Addr= 0.0.0.0 NAT= Full Feature Address Mapping Set= 1 Telco Options: Transfer Type= 64K Multilink= Off Idle Timeout= 300 Press ENTER to Confirm or ESC to Cancel:

Case 3 (General case, with 3 IGAs)

In this example, user has to configure 4 rules, 2 for one-to-one mapping (one-to-one for both incoming packets and outgoing packets), one for outgoing packets, and the other for incoming packets. The address mapping set has to be configured as following (use set 1 in this case).

Menu 15.1.1 - Address Mapping Rules Set Name= test Idx Local Start IP Local End IP Global Start IP Global End IP Type --- --------------- --------------- --------------- --------------- ------ 1. 192.168.1.10 202.132.50.1 1-1 2. 192.168.1.11 202.132.50.2 1-1 3. 0.0.0.0 255.255.255.255 202.132.50.3 M-1 4. Server Set= 1 202.132.50.3 Server 5. 6. 7. 8. 9. 10. Action= Edit , Select Rule= 0 Press ENTER to Confirm or ESC to Cancel:

Another, the server set 1 has to be set as following.

Menu 15.2.1 – Multiple Server Configuration Port # IP Address ------ --------------- 1. Default 192.168.1.20 2. 0 0.0.0.0 3. 0 0.0.0.0 4. 0 0.0.0.0 5. 0 0.0.0.0 6. 0 0.0.0.0 7. 0 0.0.0.0 8. 0 0.0.0.0 9. 0 0.0.0.0 10. 0 0.0.0.0 11. 0 0.0.0.0 12. 0 0.0.0.0 Press ENTER to Confirm or ESC to Cancel:

In this case, menu 11.3 has to be configured as following,

Menu 11.3 - Remote Node Network Layer Options Rem IP Addr: 202.132.155.100 Rem Subnet Mask= 255.255.255.0 My WAN Addr= 202.132.154.100 NAT= Full Feature Address Mapping Set= 1 Metric= 2 Private= No RIP Direction= Both Version= RIP-2B Press ENTER to Confirm or ESC to Cancel:

- CLID Support

The CLID support depends on country code, but in some countries the CLID formats are

unknown, the user could choose the CLID format by AT command "ATYn", please add in

initial string by CI command “isdn ini set Yn”, where

n=0-6  *0   Caller ID signaling option ( for CLID hardware)

Y0          by country default

Y1          DTMF, prior to ring

Y2          ETSI FSK, during ring

Y3          ETSI FSK, prior to ring.

Y4          Bellcore FSK, during ring

Y5          Bellcore FSK, prior to ring

Y6          DTMF, during ring

Note that caller ID is only supported on P100IH plus (with back label marked “ZyIH01”). Please

reboot system to enable after setting commands. Issuing CI command "isdn ini clear" may clear the

added initial settings.

There’re 2 output patterns that could be selected for FSK format, one bit could be used :

For MDMF format, it’s the default and not necessary to change.

For SDMF format, add further setting in initial command "isdn ini set YnS122.6=1".

## Bug fixes:

- ISDN layer 2 can’t link up problem with some switch.

- Connect two remote nodes with SUA only, the packets only gets through on the node that got dialed last.

- No tone is generated for call waiting if there’s 3rd party incoming phone call.

- The setting date in SMT menu 24.10 is decreased by one in SMT menu 24.1.

- Callback CLID mismatch problem.

- Fax transmission through POTs may cause router to crash.

- Siemens switch compatibility issue with U interface.

- In SMT menu 24.10, user can’t reset dial-in budget.

- Fix bug that BACP can’t be disabled from CI command “ppp lcp bacp off”.

## Known Bugs:

- For DSS-1 version, Prestige may stop placing outgoing data calls after Call Waiting/Call Hold/ Call Retrieve scenario if both of POTS ports are assigned the identical phone number. When it happens, the B-channel status shown on Menu 24.1 is wrong.

- Prestige performance will be degraded if there exists a telnet session in Menu 24.1 via LAN at the same time.

- For old P100IH models with 0.5M DRAM (could be check by ATME in debug mode), you may have difficulties to backup configuration due to system unable to allocate enough memory to perform the function. The workaround is to use PCT to perform backup function in this case.