ZyXEL Communication Co.

| Subject: ADSL DMT USB Device Driver Release Notes: Driver Release 1.11.0007Q Annex: A Control Panel Release 3.1.10 Installer Version: 1.16.1 For: ADSL DMT USB: REFP-7070-174-102 (GDS-0102-02) & REFP-7070-174-108 (GDS-0108-01) | Date: October 2, 2001 |
| --- | --- |

Introduction:  These notes describe the 1.11.07Q production release for the USB device driver.

DSP Firmware Version: This release is built on DSP firmware version Q43 (Production Release 5.00.50).

Default setting :

- Other service Provider : making a selection by your need

- Support RFC1483 bridged mode and Routed mode when select LAN driver

- Support RFC2364 PPPoA when select WAN driver

- ZyXEL WAN :

- VPI/VCI = 8/35

- Encapsulation = RFC 2364 PPPoATM NULL Encapsulation

- Modulation = Multimode

- ZyXEL LAN :

- VPI/VCI = 8/35

- Encapsulation = RFC 1483 IPoATM Bridged LLC Encapsulation

- Modulation = Multimode

New features: (detailed explanation of new feature)

- “True” multimode for the T1.413, G.DMT, and G.lite standards

- MTU settings for the WAN driver

- Log file for the installer

- Additional Retrain Criteria

- BER test for Spain Telefonica

Problems corrected in this release:

(Number denotes MR #)

| 2677 | Retrain issue with Alcatel DSLAM |
| --- | --- |
| 2834 | GSI up/down icon does not re-appear after Hot-Plug |
| 3043 | ADI918-2 minimum margin feature does not work |
| 3199 | Installation support for Windows XP |
| 3224 | RFC1577 driver does not support DHCP |
| 3326 | PPPoE WAN driver causes blue screen in Win98SE and WinMe |
| 3345 | Performing uninstall via control panel doesn't work in some cases |
| 3359 | Systray Icon cannot be disabled |
| 3366 | Dying gasp does not work to Alcatel DSLAM |
| 3438 | Run on Exit does not work (Installer) |
| 3367 | NDISWAN bindings after un installing the driver |

Problems corrected in previous 1.11 release (1.11.0004Q):

| 1983 | Debug Mode: Inaccurate Error reporting in Control Panel |
| --- | --- |
| 2411 | Msgsrv32 Not responding during installation |
| 2430 | Train-up time is greater than 2 minutes, if the CO-GDS is set to G.DMT |
| 2449 | Cancel button during driver installation |
| 2468 | In ATM driver, closing dial-up session causes TAPISRV error (only on WinME and rarely occurs) |
| 2631 | Setup: Ikernel.exe could not be launched |
| 2832 | Toshiba notebook not responding |
| 2852 | Training issue with TI3.2 SDSLAM. |
| 2864 | Lots of CRC errors when doing upload on certain OHCI controller |
| 2880 | Unable to bring up modem with fresh install (Sony Laptop) |
| 2886 | PC pauses for 15seconds in the middle of the uninstallation |
| 2888 | Updating the driver from the installer does not work properly |
| 2902 | Control Panel debug mode can not be disabled |
| 2903 | Multi-Language support |
| 2904 | Control Panel Size (Large fonts shifts the logo around) |
| 2932 | Error message for WAN driver under Win98SE (Configure button does not display the rate information) |
| 2954 | Modem generates a lot of AAL5 CRC errors (2 session) |
| 2964 | Cell counter rolls over to negative value |
| 2974 | Does not report correct attenuation by the firmware. |
| 2982 | Fail to login with BRAS/IPSS |
| 2989 | PPPoE driver unable to browse some website like real.com |
| 3002 | SNR Margin not reported for Lucent Stinger |
| 3004 | Dying gasp not working for Alcatel DSLAM |
| 3025 | Packet size less than 64 bytes for the LAN driver does not work. |
| 3042 | instdll.dll file does not get deleted by remove |
| 3048 | ADI918-1 drops connection after data mode |
| 3082 | No “Restart the computer” message when driver is installed |
| 3097 | 0dB in the CPL does not give BER of 1E-7 |
| 3100 | Modem should drop the line and retrain when DMT-CRC |
| 3199 | Installation support for Windows XP |
| 3132 | Request for customization of two DSL Installer |
| 3133 | Disableing CPL debug mode when "AllowCplDebugMode" line is removed in "wwdslcfg.ini" |
| 3170 | Alcatel HH3CAB.455 in G.dmt mode training issue during 1-5Kft |
| 3275 | Installer settings aren't read by driver in Win2k |

Known limitations and problems with this release:

Hardware

- On dual processor machines running Windows XP, the USB driver does not go into data mode.

Installer

- On any OS, during the installation process, clicking “No” in the “Software License Agreement” window of the Installer gives the error message “Installshield can not call dll function GetDlgItem”.  This error will not affect functionality.

- On a Win2K OS, during the installation process, clicking “No” in the “Digital Signature” window causes the installer to stop the installation and leaves ZyXEL files on the PC.  This is not an issue if Windows 2000 CAT file is used.

- On Windows XP, the file GSI.exe is left in the system after un-installing the driver.

- The String “Encapsulado VC IPoATM encaminado RFC 1483RFC” is still showing the RFC at the end for the Spanish language.

Control Panel & System Tray Icon

- Debug Mode: Closing the control panel clears all the physical layer error counters.

- On the initial boot, if the GSI System Tray icon is used to open the Control Panel, clicking (either right or left clicking) on the System Tray icon will not close the Control Panel.  This only seems to occur once, on initial reboot.  It does not have any negative effect on the Control Panel or the modem’s functionality.

- Debug Mode: SNR margin displayed in the Control Panel is not updated unless the window is closed and then re-opened.  If the window is left open the SNR margin will not update.

- Two Control Panel’s can sometimes be launched simultaneously from the system tray icon in Windows ME.  This problem is very rare.

- The last letter of the strings “Trasmissione” and “Ricezione” in the Italian language are being truncated in the Control Panel.

- Ctrl + F1 combinations buttons do not launch the Debug mode of the Control Panel on Korean Windows ME and Simplified Chinese Windows 2000. The mouse-click combination does work.

LAN

- On a Toshiba Protégé laptop running Win2K, FTP download of a 51megs file causes approximately 27-31 AAL5 errors to occur.  This does not have a large impact on FTP throughput.

WAN

- The PC can be put into a state where a dial-up networking (DUN) connection cannot be established any more without the need of rebooting the system. This occurs when a DUN has been established with either the PPPoA or PPPoE drivers on any O/S.  If the USB cable is unplugged from the device while a DUN connection window is open, or simply clicking on the disconnect button to terminate the open DUN connection, a Microsoft window pops up stating that DUN connection has been lost.  The DUN window appears with the disconnect button disabled. If the USB cable is plugged back in, the user will not be able to make a DUN connection until the PC is rebooted. This is a Microsoft Windows issue.

WHQL

- The WHQL USB serialization test on Windows ME causes lock up or a blue screen with the LAN and WAN drivers.  This problem is intermittent.

# New Features:

# Multimode Support

Introduction

The “true” multimode feature is implemented in the transceiver. Due to the feature, transceiver would toggle between T1.413 (Multi-vendor) and G.DMT/G.Lite standard every 2 seconds till successful “handshake” to DSLAM.

Due to this new feature, the automode parameter in the registry provided by the driver is obsolete.

Automode = Obsolete parameter for the driver

# Maximum Frame Size Setting

Introduction

Some routers can’t fragment and process a frame with length of greater than 1492 bytes because the “Don’t fragment” field is set to one in the frame. Therefore it will not allow some websites to be viewed.

The MTU (maximum Transfer Unit) needs to be changed, which is partially handled by the driver and DUN (Dial-up networking).  A new parameter has been added to the registry to control the MTU size in the driver.

Registry Settings

A new parameter has been added to the registry to control the maximum frame size (or maximum Transfer Unit) for the packet in the driver. The proper setting is specified in the wwdslcfg.ini file.

MaxFrameSize = 1492 (Default)

# Log file for the installer

Added an error and trace logging service.  This service logs debug messages to a file called wwdslcfg.log in the windows directory, and may be useful when trying to debug a field problem.  To enable this feature, add Debug=ERROR or Debug=TRACE to the SystemInfo section of the wwdslcfg.ini file.  TRACE will display more information than ERROR.

# Retrain Criteria

Two additional criteria are added to the retrain function. The previous retrain function for the USB driver was based only on the monitoring of the GTI_OVERALL_FAILURE item in the GTI_FAILURE status. The retrain function would retrain if the GTI_OVERALL_FAILURE is non-zero value, which indicates one or more failures has occurred for 2.25 seconds or more.

Additional Criteria

It is important to monitor the CRC errors as well and retrain if high number of CRC errors has occurred. CRC errors can monitor by using the GTI_COUNTER status. Guidelines for detecting unacceptably high CRC error rates are as follows:

- CRC error rate exceeds 25 per second for 3 consecutive seconds. This detects a near total loss.

- CRC error rate exceeds the number of bytes per DMT symbol per minute for five consecutive minutes. This detects a slow deterioration in performance. (Example, at 8064kb/s this means detecting more than 252 CRC errors per minute for five consecutive minutes).

Registry Option

The retrainEnable parameter in the registry can disable and enable the retrain criteria.

| Parameter | Values | Description |
| --- | --- | --- |
| RetrainEnable | 0: Disable | Monitor the retrain criteria but do NOT retrain the DSL line |
|  | 1: Enable (Default) | Retrain the DSL line based on the retrain criteria |

# BER Testing for Spain Telefonica

Various service providers worldwide use different methods for calculating the Bit Error Rate (BER) for a given DSL connection.  The description below pertains to the proper BER calculation that is needed to pass the requirements at Spain Telefonica.  The new parameter outlined in the table below should be used for Spain Telefonica only.  The parameter should be left in the default state (0) for all other opportunities.

The SNR margin was being reported by averaging the SNR margin per bin for all of the bins (average of 100% of the total bins). When injected with certain noise the SNR margin would still report positive SNR margin (even with half of the bin are negative). This caused a potential problem when calculating the BER.

The “new” SNR margin is calculated by taking 10% of the total bins to average the minimum of the SNR margin per bins. To be less aggressive on the data rate, the coding gain is also lowered by 2dB, which is factor in the SNR margin calculation.

Due to the nature of the fix, the “new” SNR margin is provided to the customer with a special command in the registry.

Registry Settings

To set the parameter in the registry:

| Parameter | Value | Description |
| --- | --- | --- |
| RxBinAdjust | 0xF0 | Reports “new” SNR margin (10% of the bins) |
|  | Others(default=0) | Report SNR margin using 100% of the bins (same as before) |