# Comparative

## 16-bit complex FFT

STMicroelectronics ARM9E-based STR91x

|  | Cycle count | at 96 MHz | at 60 MHz | at 30MHz |
| --- | --- | --- | --- | --- |
| 64-point | 2,701 | 28.14 us | 45.02 us | 90.03 us |
| 256-point | 13,740 | 143.12 us | 229.00 us | 458.00 us |
| 1024-point | 68,534 | 713.90 us | 1.14 ms | 2.28 ms |

Microchip dsPIC

|  | Cycle count | at 40 MHz | at 30 MHz |
| --- | --- | --- | --- |
| 64-point | 3,739 | 93.5 us | 124.63 us |
| 256-point | 19,055 | 476.4 us | 635.17 us |

## 16-bit FIR Filter

STMicroelectronics ARM9E-based STR91x

STMicroelectronics uses another algorithm to compute its FIR Filter. It is not comparable because it is made to compute a signal in real-time while ours compute a whole block of data and so is not optimized the same way as ours.

Comparatively, to compute a 24-TAP filter of a signal of 64 points, ST algorithm would take approximately 1,813*(64-24+1) = 74,333 cycles, while ours takes only 2,439 cycles.

Microchip dsPIC

Contrariwise to STMicroelectronics, Microchip uses the same algorithm as ours. For the same result, it takes about 33% cycles less than ours. Indeed, the FIR algorithm is based on the “MAC” instruction and dsPIC owns a very efficient one.

In only one cycle, this instruction can read two 16-bits data from memory, update the memory pointers and perform a MAC instruction.

(cf. “Microchip dsPIC – Section 2. - Instruction Set Details” page 35) (http://ww1.microchip.com/downloads/en/DeviceDoc/sect2.pdf)

## 16-bit IIR Filter

STMicroelectronics ARM9E-based STR91x

The following results are for a 4th order filter.

| Number of Taps | Cycle count | at 96 MHz | at 60 MHz | at 30MHz |
| --- | --- | --- | --- | --- |
| 24-tap | 2,248 | 23.42 us | 37.47 us | 74.94 us |
| 48-tap | 4,396 | 45.79 us | 73.27 us | 146.54 us |
| 72-tap | 6,544 | 68.16 us | 109.07 us | 218.14 us |

Microchip dsPIC

Formula: 46+N*(16+7*M) where M is the order of the filter and N the number of points to process.

Advantages of the AVR32-DSPLib compared to the others:

- It can use any fixed-point Q-Formatted numbers.

- It is compatible with any platforms that are supported by GCC and IAR.

- It can be customized with several algorithm optimization options according to your needs.

Code optimization with DSP Instructions:

This graphics shows the difference between the code optimized with DSP Instructions for the avr32-uc3 (■ avr32 uc3 optimized) and the code without (■ generic). On the avr32-uc3, we gain from 100% to 200% of speed execution.

List of currently implemented functions:

Complex FFT, FIR filter, convolution, partial convolution, sine/cosine, sinusoidal/cosinusoidal generator, vector copy, zero padding.

Performance comparison:

IIR Filter performance comparison